SPECIFICATION OF THE IS-136 Rev A CS

FIELD TEST DISPLAY

Part:	DAMPS_CS_SW:DAMPS_SPEC

Item:	DAMPS_CS_SW:IS136_FT_DISPLAY_SPEC.BASE-SPEC

Filename:	IS136_FT_DISPLAY_SPEC.DOC

Document code: DSS00056

Copyright ©  1996-1998.  Nokia Mobile Phones

Change History:

VERSION	: 7     NON-DRAFT	03-FEB-98	Petri Katekeetta

REASON	: Inspection

REFERENCE	: -

DESCRIPTION: Editorial work incl. some clarifications. Modifications in displays 4 and 5.

VERSION	: 6     DRAFT		27-OCT-97	Petri Katekeetta

REASON	: Updating of displays 4 and 5.

REFERENCE	: -

DESCRIPTION: Alphatag comment added, fasc changed to IRC etc.

VERSION	: 5     DRAFT		20-AUG-97	Mika Kunto

REASON	: Because of audio testing

REFERENCE	: -

DESCRIPTION: Added voice codec detection into the Display 2 (TCH mode).

VERSION	: 4     NON-DRAFT	20-FEB-97	Markku Halonen

REASON	: BER moved to Display 1

REFERENCE	: -

DESCRIPTION: BER moved to Display 1 and audio state moved to Display 2

VERSION	: 3     NON-DRAFT	18-DEC-96	Markku Halonen

REASON	: Inspected

REFERENCE	: -

DESCRIPTION: See inspection minutes in HD960 ADB, ref. MH96121849439

VERSION	: 2     DRAFT		13-DEC-96	Ilpo Mattila

REASON	: Added new DISPLAYS 5 and 6.

REFERENCE	: -

DESCRIPTION: Added displays 5 and 6 to provide information related to Intelligent Roaming

(IR) and Intelligent Roaming Database (IRDB).

VERSION	: 1     DRAFT       	10-DEC-96      Markku Halonen

REASON	: New document

REFERENCE	: -

DESCRIPTION: The cellular specific displays are moved from HD861_FIELD_TEST_DISPLAY_SPEC.TXT in 	   HD861_SYSENG_CMS. Added new displays for dualband and non-public mode functions.

1. VOCABULARY

2. GENERAL

3. THE CONTENTS OF THE FIELD TEST DISPLAYS

3.1 DISPLAY 1

3.2 DISPLAY 2

3.3 DISPLAY 3

3.4 DISPLAY 4

3.5 DISPLAY 5

3.6 DISPLAY 6

3.7 DISPLAY 7

# 1. VOCABULARY

ACCH      Analog Control CHannel

AVCH      Analog Voice CHannel

DCCH      Digital Control CHannel

DTCH      Digital Traffic CHannel

# 2. GENERAL

This document describes the field test displays for IS-136 revision A Cellular System.

See the product specific guide how to enable the field test mode and how to get different displays on screen.

# 3. THE CONTENTS OF THE FIELD TEST DISPLAYS

There are two displays for the following modes

- ACCH Mode

- DCCH Mode

- AVCH Mode

- DTCH Mode

The mode is recognized by the field test software and the contents of display will change according to the mode.

Other displays can be selected by the user.

## 3.1 DISPLAY 1

ACCH mode:

|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | R | s | s | i |  |  |  | D |  | d |
| 1 | B | c | h | a | n |  | P | w |  | A |
| 2 | C | S | - | s | t | a | t | e |  |  |
| 3 |  |  |  |  |  |  |  |  |  |  |

AVCH mode:

|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | R | s | s | i |  |  |  | s |  | d |
| 1 | B | c | h | a | n |  |  | l |  | a |
| 2 | C | S | - | s | t | a | t | e |  |  |
| 3 |  |  |  |  |  |  |  |  |  |  |

DTCH mode:

|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | R | s | s | i |  | D | V | C |  | S |
| 1 | B | c | h | a | n |  | l |  | B | e |
| 2 | C | S | - | s | t | a | t | e |  |  |
| 3 |  |  |  |  |  |  |  |  |  |  |

DCCH mode:

|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | R | s | s | i |  | D | V | C |  | S |
| 1 | B | c | h | a | n |  | P | w |  | A |
| 2 | C | S | - | s | t | a | t | e |  |  |
| 3 |  |  |  |  |  |  |  |  |  |  |

rssi	= received signal strength in dBm. Minimum is -113 dBm

and maximum -51 dBm. Value is changed with 2 dBm steps.

D       	= DCC (Digital Color Code) on ACCH, 0...3, '-' = not locked to ACCH

s       	= sat color  0..2

DVC     	= DVCC 1 - 255, '---' = not locked to DCCH

d       	= data receiving status

0 = no sync/ no data signal

1 = syncronised, not able to read

2 = reading, correcting

3 = reading, no corrections

B 	= Band,

a, b = low band

A, B, C, D, E , F = high band

chan    	= channel  0001...1999

l       	= tx level VMAC,

0...10 on DTCH

0...7 on AVCH

- = Tx off

Be      	= BER in %

a       	= audio state (0 = OFF, 1 = ON)

A       	= authentication status

0 = authentication disabled

1 = authentication enabled

- = not locked to control channel

Pw      	= Maximum Access Power level on control channel

MS_ACC_PWR (0...10) on DCCH

CMAC (0...7) on ACCH

- = not locked to the channel

S       	= slot number 1...3

CS-state	= state of cellular

OOR         		(ACCH: Out of range)

SCAN_PDCH   	(ACCH: Scan Primary dedicated control channels)

SCAN_SDCH   	(ACCH: Scan Secondary dedicated control channels)

SCAN_PCH    		(ACCH: Scan Paging channels)

IDLE        		(ACCH: Idle)

ACCESS      		(ACCH: Access)

VCH         		(AVCH: Confirm voice channel)

CONVERSAT  	(AVCH/DTCH: Conversation)

TCH         		(DTCH: Confirm traffic channel)

SCAN_LOCK  		(DCCH: Scanning and locking of DCCH)

DCCH_SEL    		(DCCH: DCCH selection)

CAMPING     		(DCCH: Camping)

REGISTR     		(DCCH: Registration proceeding)

WAIT_ORDER	(DCCH: Waiting for order)

ORIGINAT    		(DCCH: Origination proceeding)

RESELECT    		(DCCH: Reselection)

ORIG_SMS   		(DCCH: Originated SMS proceeding)

TERM_SMS    		(DCCH: Terminated SMS proceeding)

SSD_UPD		(DCCH: SSD Update)

## 3.2 DISPLAY 2

ACCH mode:

|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | M | o | d | e |  |  |  |  |  |  |
| 1 | X | S | T | B | Y |  |  |  |  |  |
| 2 | S | I | D | . | . |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |  |  |  |

mode   	= current channel mode, ACCH text is shown

XSTBY   	= text XSTBY is shown when extended standby is on

otherwise '-----' is displayed.

SID     	= system identification 0...32767,

'-' = not locked to ACCH

AVCH mode:

|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | M | o | d | e |  |  |  |  |  |  |
| 1 |  |  |  |  |  |  |  |  |  |  |
| 2 | S | A | T | . | . |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |  |  |  |

mode	= current channel mode, AVCH text is shown

SAT     	= SAT validity measured by DSP 0...32767

DTCH mode:

|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | m | o | d | e |  | D | T | X |  |  |
| 1 | E | N | C | R |  | C | I | P | R |  |
| 2 | d |  | a |  |  | T | A |  | S |  |
| 3 | C | O | D | E | C | . | . | . |  |  |

mode    	= current channel mode, DTCH text is shown

DTX     	= text DTX is shown when DTX is on,

otherwise '---' is displayed

ENCR    	= text ENCR is shown when message encryption is on,

otherwise '----' is displayed

CIPH    	= text CIPH is shown when voice ciphering is on,

otherwise '----' is displayed

d       	= data receiving status

0 = no sync

1 = synchronised

a       	= audio state (0 = OFF, 1 = ON)

TA      	= last received time alignment value 0 - 30

S	= service code

0 = digital speech call

1 = async data call

2 = G3 FAX call

CODEC...	= Show which voice codec is detected,

text -EFR- is shown when EFR is on,

text VSELP is shown when VSELP is on

DCCH mode:

|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | M | o | d | e |  | M |  | C |  |  |
| 1 | F | B |  | E |  | S | P |  | p | c |
| 2 | n | e | t |  | S | I | D | . | . |  |
| 3 |  |  |  |  |  |  |  |  |  |  |

mode    	= current channel mode, DCCH text is shown

M       	= MAX_SUPPORTED_PFC, maximum paging frame class supported by

the network, 1...8

C       	= Current PFC (Paging Frame Class 1...8)

FB      	= Number of F-BCCH slots

E       	= Number of E-BCCH slots

SP      	= Number of SPACH slots

pc      	= PCH subchannel

net     	= Supported network types, bit map:

1 = supported,

0 = not supported

xxx

  Residential

 Private

Public

Example: if the network type of the current channel is private,

public networks supported and residential not then 110 is shown on the display.

SID     	= system identification 0...32767,

'-' = not locked to DCCH

If the phone is in DCCH mode but not locked to the channel (scanning)

then '-' characters will be shown on display.

## 3.3 DISPLAY 3

This display shows information in DCCH mode only, in other modes (ACCH, DTCH, AVCH) following display will be shown:

|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | D | C | C | H |  | n | o | t |  |  |
| 1 | a | c | t | i | v | e |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |  |  |  |

In DCCH mode the display contains:

|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | R | S |  | S | S |  | S | I |  | b |
| 1 | N | A |  | N | D |  |  |  |  |  |
| 2 | M | A |  | M | D |  | M | O |  |  |
| 3 |  |  |  |  |  |  |  |  |  |  |

RS      	= RSS_ACC_MIN, minimum received signal strength required to access the cell,

0...31

SS      	= SS_SUFF, minimum signal strength deemed sufficient for a candidate

control channel to be considered for control channel

reselection.

SI      	= SCANINTERVAL, interval between consecutive signal strength measurements

(in hyperframes) 1...16

b       	= Access burst size

0 = normal length RACH bursts

1 = abbreviated length RACH bursts

NA      	= Number of Analog neighbors 0...24

ND      	= Number of Digital neighbors in low band  0...24

MA	= Number of Analog neighbors in multihyperband 0...24

MD	= Number of Digital neighbors in multihyperband 0...24

MO	= Number of neighbors in other band 0...24

If the phone is in DDCH mode but not locked to the channel (scanning)

then '-' characters will be shown on display

## 3.4 DISPLAY 4

This display shows information in non-public mode.

|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | S | I | D | . | . |  |  | t |  |  |
| 1 | N | N |  | N | T |  | N | R |  |  |
| 2 | A | l | p | h | a | t | a | g | . | . |
| 3 |  |  |  |  |  |  |  |  |  |  |

SID	= SID/PSID/RSID value where registrated, 0 ... 61439

t	= network type where registrated

4 = public

2 = private

1 = residential

0 = ACCH

'-' = not registered

NN	= Number of PSIDs/RSIDs in NAM area

NT	= Number of PSIDs/RSIDs in Test Registration area

NR	= Number of PSIDs/RSIDs in Registration Accept area

Alphatag	= Alphatag where registrated or last registrated (first 10 characters only)

## 3.5 DISPLAY 5

This display is for dual-band phones only.

It shows information from Intelligent Roaming.

|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | S |  | b | a | n | d | o | r | d | e |
| 1 | N | C |  | N | P |  | I | R | C |  |
| 2 | R | S | C | O |  | R | S | L | O |  |
| 3 |  |  |  |  |  |  |  |  |  |  |

S	= Service Provider Type - On Service

System under scanning - No Service

Possible values when On Service:

H = Home,      P = Partner,       F = Favored

N = Neutral,   X = Forbidden

When No Service Condition exists

and scanning is ongoing the system

which phone is currently scanning will be

displayed.

Possible values:

A, a, B, b, C, D, E or F.

No Service and no Scanning (OOR):

“-” will be displayed.

bandorde	= Scanning order of the given bands in IR-database.

Examples: AaBbCDEF,  baD,  FEaAD

IRC 	= IR control. Values 0 or 1.

NC	= Number of cellular (800 MHz) probability blocks to scan (0...16)

NP	= Number of PCS (1900 Mhz) sub-bands to scan (0...7)

RSCO	= Rescan Count. Time in hyperframes* how long to wait before searching

for acceptable SP on the last used acceptable band.

RSLO	= RESCAN_LOOP. Tells how many times to search the last

used acceptable band before doing a wide band scan.

* 1 hyperframe = 1,28 seconds

## 3.6 DISPLAY 6

This display is for dual-band phones only.

It shows information from Intelligent Roaming Database.

|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | P | S | C |  | P | S | D |  |  |  |
| 1 | F | S | C |  | F | S | D |  |  |  |
| 2 | X | S | C |  | X | S | D |  |  |  |
| 3 |  |  |  |  |  |  |  |  |  |  |

PSC	= Number of Partner SOCs.

Values: 0 - 82.

PSD	= Number of Partner SIDs.

Values: 0 - 82.

FSC	= Number of Favored SOCs.

Values: 0 - 82.

FSD	= Number of Favored SIDs.

Values: 0 - 82.

XSC	= Number of Forbidden  SOCs.

Values: 0 - 82.

XSD	= Number of Forbidden SIDs.

Values: 0 - 82.

## 3.7 DISPLAY 7

This display is empty and can be used to see the current user interface display.

|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |  |  |  |