Dial-Res.com  v. 2.00		By J.E.Arkay 1996 after Roger Schlafly 1990

Resident program to read phone numbers on the screen and dial them with a modem.

The screen must be in a text mode.

In Win 95, it works in PMS in a DOS window, and in any DOS program run from

PMS; but it does not work in a DOS prompt window !

However, Win 95 is so squirrly it's worth trying anything.

You can create a text file listing names and phone numbers, or dial directly

from any text-based database or spreadsheet.

Run  DIAL-RES /n  at a DOS prompt, where n is your COM port number (1 thru 8).

Default is COM1.  You can make a .BAT file to run it with your COM port,

or if you use PMS you can set it up as a menu item.

Once installed, to dial a phone number on the screen in any text program,

hit Alt-D.

The cursor will change shape, and you can use the arrow keys to position it

at the start of a phone number.

The Home, End, Ctrl-Home / PgUp, and Ctrl-End / PgDn keys also move the cursor.

The number must all be on one line.

When the cursor is at the beginning of a phone number, press Enter.

You should hear the modem dialing.  Esc will abort dialing.

When it is finished dialing, pick up the telephone and press the space bar or

ESC to hang up the modem.

You can repeat the last dialing with Alt-R.

If when you hit Enter the cursor just disappears after a second, you probably

didn't set the correct COM port.  Unload the program by rebooting (or hit

Ctrl-Alt-Ins or End if you're running PMS).  Then fix the .BAT file.

If the cursor remains but the modem doesn't dial, hit ESC and try it again

before you give up.

A phone number is a contiguous string made up of "1234567890,-()*#".

So, for example, a comma (which is usually a pause) will be sent to the modem.

Leading garbage is skipped.

It sends ATDT followed by any number it finds, then a CR.

The phone line must accept tone dialing (but you could patch it for ATDP).

You can patch the .COM file with a byte editor.  To change modem commands

search for the string "This space for modem dial commands".

The hot keys follow "Dial and Redial".  They are now Alt-D = 00 20 and

Alt-R = 00 13.  To use, for example, F10 patch these to 00 44.

The only place I ever saw these Extended key codes listed is the old Norton's

Programmer's Guide to the IBM PC.  The freeware @LAST.COM also shows the

key scan codes.

It uses only interrupts 10h, 14h, 16h, and 21h.  It fixes a bug with my USR

33.6 modem on COM4.  For some reason my BIOS put the port address in the

wrong location in the BIOS data area at 40:0000, preventing Int 14h and

MODE from recognizing the modem.  See the .ASM source file included.

If you post this PLEASE INCLUDE the .ASM file.

NOTICE: WE DIDN'T TEST THIS VERY MUCH AND WE ACCEPT NO RESPONSIBILITY FOR IT.