This is the document file for BC-07 v1.00

Copyright (c) 1997 by M. Nimal Gunarathne

�����������������������������������������

Welcome to the world of shareware.

Dear friend, This is a very special, unique program.  You are very

lucky to come across this.   Please use,  and give it to everybody

and make them lucky too. Thank you.

Short description ( If you are too busy. )

�����������������

This is a unique program,  designed to  show you,  how to cut many

pieces of different lengths required, from the standard lengths of

materials such as bars, beams, pipes, timber etc. without wasting.

Detailed description

��������������������

Contents             [1]  What is it ?.

��������             [2]  How to use ?.

[3]  Who needs it ?.

[4]  How to order ?.

[5]  Other.

[1]. What is it ?

�����������������

It is a unique program  that  saves you MONEY,  MATERIAL and TIME.

You will realize it's power, and the importance at first instance.

I have named it as "BC", short for BEST CUTTER.  It's very useful.

Let us consider a real example.   You are constructing your house.

You have estimated the required amount  of some items like timber,

steel, pipes etc. The required amounts of steel are given below.

For different purposes, different lengths

��������������������Ŀ   of steel bars are  required.  The  length

� Length  �  Number  �   and  the number required from each length

� (Units) � required �   are given in the table. ( Assume the dia-

��������������������Ĵ   meter is same for all listed. )

�         �          �

�  2.55   �    723   �   Let us assume that the length of standard

�  1.95   �    839   �   steel bars available is 6 Units.  Now you

�  1.65   �    694   �   may  wish to know, how many lengths to be

�  1.35   �    593   �   bought, and  how to  cut the above pieces

�  0.90   �    843   �   from  standard lengths,  without wasting.

�  0.60   �   1200   �

�  0.45   �    511   �   At  this  moment,  you should realize the

�         �          �   importance  of   handling   this  problem

����������������������   methodically to avoid possible  waste  of

money and material.

If you start analyzing and guessing the possibilities with a paper

and  pencil, you  might spend  several  days to find the solution.

( I doubt it very much even if you are a very lucky Math Wizard!.)

Most of the people will not find the answer. Therefore I worked on

finding an easier and quicker way of solving this kind of problems

using computers.

The result is BC-07.   When you run BC-07 and enter the  necessary

details the solution will be written to a file named BC07 as given

below.

--------------------------   PROBLEM   ---------------------------

Length of the standard bars available  =    6     Units.

It is required to cut     723  No. of      2.55  Unit lengths,

839  No. of      1.95  Unit lengths,

694  No. of      1.65  Unit lengths,

593  No. of      1.35  Unit lengths,

843  No. of      0.90  Unit lengths,

1200  No. of      0.60  Unit lengths,

511  No. of      0.45  Unit lengths,

with a MINIMUM waste.

---------------------------  SOLUTION  ---------------------------

Standard length =    6

No.        723    839    694    593    843   1200    511

Len.      2.55   1.95   1.65   1.35   0.90   0.60   0.45    Waste

-----------------------------------------------------------------

361  x      2      .      .      .      1      .      .        0

1  x      1      1      .      .      1      1      .        0

419  x      .      2      1      .      .      .      1        0

91  x      .      .      3      .      .      1      1        0

1  x      .      .      2      2      .      .      .        0

147  x      .      .      .      4      .      1      .        0

1  x      .      .      .      3      1      1      1        0

80  x      .      .      .      .      6      1      .        0

88  x      .      .      .      .      .     10      .        0

-----------------------------------------------------------------

1189                                                         0.00

Total bars  =   1189    (        7134.00 Units )

Total waste =      0.00 Units (  0.000 Percent )

The above answer can be explained as follows.

PROBLEM  rewrites the data clearly, in descending order of length.

The first three lines of the SOLUTION is data once again,  written

horizontally.  Please note that "Number" & "Length" are written as

"No." & "Len." due to the limited space.

Let us consider the lines below the dashed line.

First row says.

Take a standard length (6 Unit) steel bar, and cut it in to,

(  2 of 2.55 Unit length   +  1 of 0.90 Unit length  ).

Do above procedure 361 times. ( Resulting a zero waste )

Second row says.

Take a standard length (6 Unit) steel bar, and cut it in to,

(  1 of 2.55 Unit length   +  1 of 1.95 Unit length  +

1 of 0.90 Unit length   +  1 of 0.60 Unit length )

Do above procedure  once.     ( Resulting a zero waste )

Third row says.

Take a standard length (6 Unit) steel bar, and cut it in to,

(  2 of 1.95 Unit length   +  1 of 1.65 Unit length  +

1 of 0.45 Unit length )

Do above procedure 419 times. ( Resulting a zero waste )

.            .             .            .             .

.            .             .            .             .

.            .             .            .             .

Proceed in a similar manner for each row    .             .

.            .             .            .             .

.            .             .            .             .

.            .             .            .             .

Last row says.

Take a standard length (6 Unit) steel bar, and cut it in to,

(  10 of 0.60 Unit length  )

Do above procedure 88 times.  ( Resulting a zero waste )

Finally you will  have  all  your  requirements  ready with a zero

waste.  Don't you think it is very convenient ?. You might find it

difficult to understand the above explanations at the  first  time

you read. Read it several times leisurely. It is quite simple.

The  best way to know  about BC-07 is to use it.  Try anything you

can imagine.   It will give  you  the best solution ever.   As the

number of different lengths increase,  the solution  will be  more

economical.  This program can handle up to   7 different  lengths.

Let us have another example.

--------------------------   PROBLEM   ---------------------------

Length of the standard bars available  =    6.50  Units.

It is required to cut     188  No. of      5     Unit lengths,

190  No. of      4.50  Unit lengths,

400  No. of      3.25  Unit lengths,

308  No. of      1.75  Unit lengths,

181  No. of      1.50  Unit lengths,

483  No. of      1.25  Unit lengths,

323  No. of      0.75  Unit lengths,

with a MINIMUM waste.

---------------------------  SOLUTION  ---------------------------

Standard length =    6.50

No.      188    190    400    308    181    483    323

Len.       5   4.50   3.25   1.75   1.50   1.25   0.75    Waste

---------------------------------------------------------------

181  x    1      .      .      .      1      .      .        0

7  x    1      .      .      .      .      .      2        0

190  x    .      1      .      .      .      1      1        0

200  x    .      .      2      .      .      .      .        0

102  x    .      .      .      3      .      1      .        0

1  x    .      .      .      2      .      .      4        0

47  x    .      .      .      .      .      4      2        0

3  x    .      .      .      .      .      1      7        0

---------------------------------------------------------------

731                                                       0.00

Total bars  =    731    (        4751.50 Units )

Total waste =      0.00 Units (  0.000 Percent )

If you think that the above examples are trivial,  let us consider

little more difficult problems.   Please note, that you cannot use

BC-07 to verify the results. You will need a registered version.

Example - 01

������������

--------------------------   PROBLEM   ---------------------------

Length of the standard bars available  =    4.50  Units.

It is required to cut     901  No. of      2     Unit lengths,

600  No. of      1.70  Unit lengths,

348  No. of      1.60  Unit lengths,

671  No. of      1.40  Unit lengths,

320  No. of      1.20  Unit lengths,

590  No. of      1.10  Unit lengths,

700  No. of      0.90  Unit lengths,

401  No. of      0.80  Unit lengths,

10  No. of      0.70  Unit lengths,

330  No. of      0.60  Unit lengths,

with a MINIMUM waste.

---------------------------  SOLUTION  ---------------------------

Standard length =   4.50

No.      901  600  348  671  320  590  700  401   10  330

Len.       2  1.7  1.6  1.4  1.2  1.1  0.9  0.8  0.7  0.6   Waste

-----------------------------------------------------------------

401  x    1    1    .    .    .    .    .    1    .    .       0

348  x    1    .    1    .    .    .    1    .    .    .       0

152  x    1    .    .    1    .    1    .    .    .    .       0

99  x    .    2    .    .    .    1    .    .    .    .       0

1  x    .    1    .    2    .    .    .    .    .    .       0

258  x    .    .    .    2    .    1    .    .    .    1       0

1  x    .    .    .    1    2    .    .    .    1    .       0

106  x    .    .    .    .    3    .    1    .    .    .       0

27  x    .    .    .    .    .    3    .    .    .    2       0

49  x    .    .    .    .    .    .    5    .    .    .       0

1  x    .    .    .    .    .    .    1    .    .    6       0

3  x    .    .    .    .    .    .    .    .    3    4       0

-----------------------------------------------------------------

1446                                                         0.00

Total bars  =   1446    (        6507.00 Units )

Total waste =      0.00 Units (  0.000 Percent )

Example - 02

������������

--------------------------   PROBLEM   ---------------------------

Length of the standard bars available  =    8.50  Units.

It is required to cut     301  No. of      3.60  Unit lengths,

216  No. of      2.90  Unit lengths,

372  No. of      2.80  Unit lengths,

234  No. of      2.60  Unit lengths,

273  No. of      2.40  Unit lengths,

203  No. of      2.10  Unit lengths,

285  No. of      1.80  Unit lengths,

429  No. of      1.70  Unit lengths,

267  No. of      1.60  Unit lengths,

193  No. of      1.50  Unit lengths,

with a MINIMUM waste.

---------------------------  SOLUTION  ---------------------------

Standard length =   8.50

No.      301  216  372  234  273  203  285  429  267  193

Len.     3.6  2.9  2.8  2.6  2.4  2.1  1.8  1.7  1.6  1.5   Waste

-----------------------------------------------------------------

203  x    1    .    1    .    .    1    .    .    .    .       0

98  x    1    .    .    .    .    .    1    .    1    1       0

84  x    .    1    2    .    .    .    .    .    .    .       0

47  x    .    1    .    1    .    .    .    .    .    2       0

1  x    .    1    .    .    1    .    .    1    .    1       0

84  x    .    1    .    .    1    .    .    .    2    .       0

1  x    .    .    1    .    1    .    .    1    1    .       0

187  x    .    .    .    1    1    .    1    1    .    .       0

48  x    .    .    .    .    .    .    .    5    .    .       0

-----------------------------------------------------------------

753                                                         0.00

Total bars  =    753    (        6400.50 Units )

Total waste =      0.00 Units (  0.000 Percent )

Example - 03

������������

--------------------------   PROBLEM   ---------------------------

Length of the standard bars available  =   100  Units.

It is required to cut        990  No. of    65  Unit lengths,

133  No. of    31  Unit lengths,

719  No. of    24  Unit lengths,

500  No. of    21  Unit lengths,

670  No. of    18  Unit lengths,

891  No. of    16  Unit lengths,

107  No. of    15  Unit lengths,

651  No. of    12  Unit lengths,

884  No. of    11  Unit lengths,

602  No. of     7  Unit lengths,

with a MINIMUM waste.

---------------------------  SOLUTION  ---------------------------

Standard length =  100

No.      990  133  719  500  670  891  107  651  884  602

Len.      65   31   24   21   18   16   15   12   11    7  Waste

----------------------------------------------------------------

719  x    1    .    1    .    .    .    .    .    1    .      0

271  x    1    .    .    1    .    .    .    .    .    2      0

44  x    .    3    .    .    .    .    .    .    .    1      0

1  x    .    1    .    2    .    1    .    .    1    .      0

56  x    .    .    .    4    .    1    .    .    .    .      0

1  x    .    .    .    3    1    .    .    1    .    1      0

167  x    .    .    .    .    4    1    .    1    .    .      0

1  x    .    .    .    .    1    4    .    .    1    1      0

4  x    .    .    .    .    .    4    1    .    .    3      0

161  x    .    .    .    .    .    4    .    3    .    .      0

1  x    .    .    .    .    .    3    3    .    .    1      0

1  x    .    .    .    .    .    .    4    .    3    1      0

32  x    .    .    .    .    .    .    3    .    5    .      0

----------------------------------------------------------------

1459                                                           0

Total bars  =    1459   (     145900 Units )

Total waste =       0 Units ( 0.00 Percent )

Even  though  all the above examples end up with a zero waste,  it

will not be zero every time. Yes!,  But always your answer will be

the optimal,  and  the  best  possible  ever.    As  the number of

different  lengths  increase,   the  possible  combinations   also

increase.  Therefore you have a better chance of reaching the zero

limit.

The importance of the program should be obvious by now.

Normally  at  civil  construction sites a waste of 10% of steel is

considered normal. With this program  you can make it almost zero.

Just imagine the amount of money and material that can be saved.

[2]  How to use ?.

������������������

It  is  very  easy and straight forward.   Run the program by

typing BC-07 and pressing ENTER key at the DOS prompt. ( Even

if you are a WINDOWS 95 user you can run it at DOS prompt. )

The first screen, provides a brief description of the program

and advises you to read the document file, before proceeding.

Read the contents and press ENTER.

The program will clear the screen and ask the standard length

available. When you enter the above, ( Even decimals are ok.)

the length  & the number required will be asked sequentially.

You should enter these values with a space between them. (You

can have decimals for L?.) When  you have entered all the cut

sizes in the list type  "0 0"  and press ENTER. The computing

will start at this moment. An example is given below.

Standard length ? = 5.00

Enter L1  and  N1 = 2.3   40

Enter L2  and  N2 = 1.8   75

Enter L3  and  N3 = 1.5   90

Enter L4  and  N4 = 1.1  411

Enter L5  and  N5 = 0.9   69

Enter L6  and  N6 = 0.6  123

Enter L7  and  N7 =  0  0

The result will be written to the file BC07 as given below.

--------------------------   PROBLEM   ---------------------------

Length of the standard bars available  =    5     Units.

It is required to cut      40  No. of      2.30  Unit lengths,

75  No. of      1.80  Unit lengths,

90  No. of      1.50  Unit lengths,

411  No. of      1.10  Unit lengths,

69  No. of      0.90  Unit lengths,

123  No. of      0.60  Unit lengths,

with a MINIMUM waste.

---------------------------  SOLUTION  ---------------------------

Standard length =    5

No.         40     75     90    411     69    123

Len.      2.30   1.80   1.50   1.10   0.90   0.60    Waste

----------------------------------------------------------

40  x      1      1      .      .      1      .        0

35  x      .      1      1      1      .      1        0

27  x      .      .      2      1      1      .        0

1  x      .      .      1      1      2      1        0

87  x      .      .      .      4      .      1        0

----------------------------------------------------------

190                                                  0.00

Total bars  =    190    (         950.00 Units )

Total waste =      0.00 Units (  0.000 Percent )

You can print file  BC07  by issuing the following command at

the DOS prompt.

type bc07 > prn

[3]  Who needs it ?.

��������������������

As  this is a program that so many wished for and nobody had,

I think a vast  number  of  people  engaged in many different

trades / industries will need it.   All  the following people

will want it frequently in their day to day work.

Civil Engineers                     Grill makers,

Quantity surveyors                  Trellis makers

Structural Engineers                Furniture makers

Building Contractors                Roof builders

Steel, Timber & pipe dealers        Puzzle solvers

Some people, who badly need this type of solving method, may

not be aware of this program, or even may not have access to

a computer. It is nice of you to help them.

[4]  How to order ?.

��������������������

You,  are supposed to save money, material  and time with the

proper use of this program.

BC-07 can only handle a  maximum of 7 different lengths.

In real situations you will need a program  that  can  handle

more than 7 different lengths.  A powerful registered version

is the solution.

Three programs are available.  Namely BC-20, BC-30 and BC-40.

They are different in capacity.   Depending on your needs you

can order the suitable one.

�����������������������������������������������������Ŀ

�             � No. of different  �                   �

�  Program    � lengths that can  �    Price ( US$ )  �

�             � handle (maximum)  �                   �

�����������������������������������������������������Ĵ

�             �                   �                   �

�   BC-20     �        20         �        100        �

�             �                   �                   �

�   BC-30     �        30         �        120        �

�             �                   �                   �

�   BC-40     �        40         �        150        �

�             �                   �                   �

�������������������������������������������������������

����������

When you post ( Registered post is advisable. ) a Bank draft

for the  right amount (US$) to the address  given below, you

will receive a licensed copy of the program in a  3.5" disk.

( Sorry, unable to send 5.25" diskettes.  )

M. Nimal Gunarathne,

23,  First Lane,

Station Road,

Homagama,

Sri Lanka.

�������������������������������������������������������������

N.B. ->   Please  note  that you cannot use BC-20 to find the

optimum solution when you  have  more  than 20 lengths.   For

example if you have 35 different lengths,  you may feel, that

you can try 20,  and 15 lengths in two  sessions to  find the

best solution using BC-20. But, this will never be economical

as the solution, you obtain using BC-40. Again it is a waste.

Therefore select a program that covers all your needs.

�������������������������������������������������������������

[5]  Other

����������

Distribution

------------

Please distribute the file  BC-07100.ZIP  free to all others.

Permission is given to  all  to  distribute  BC-07100.ZIP  on

disk,   BBs  or  CD  provided  that it is distributed in it's

original form.

Warranty

--------

BC-07 is  provided  on  an  "AS  IS"  basis.    I  ( M. Nimal

Gunarathne  )  disclaim  all  warranties  relating  to   this

software, whether expressed or implied.  The person using the

software  bears all risk as to the quality and performance of

the software.

Do not

------

If you alter the program, the program will not work.

Do not immunize the program for virus. Program will not work.

You may not rent, lease, sell, modify, decompile, disassemble

or reverse engineer BC-07.EXE.

Comments

--------

Please  let me know your comments.  Whatever they are,  it is

nice to learn that somebody is using my program.

Please post your response to   --->  M. Nimal Gunarathne

23,  First Lane,

Station Road,

Homagama,

Sri Lanka.

or E-mail                      --->  nimal@ou.ac.lk

The world thrift day  is  31st  October.    It reminds us the

importance of avoiding the waste of money and goods.    As  a

person  born  on  the  same  day, I am happy to be able to do

something to help the concept.